SATURNE advanced use and maintenance

Following predefined GPS points autonomously and additional sensors functionalities

Using a laptop, connect to Robot_4_roues Wi-Fi network, with Robot4roues key and 192.168.1.185/24 IP address.

The embedded computer (hostname: NUC-4ROUES) can be accessed using ssh saturne@192.168.1.200 command and Robot4roues password.

Then:

cd Robot_4_roues

Update waypoints (rectangle area to scan and number of rails) in regul/src/mission.cpp (read()) and ensure:

roboteq

serial

light

nmea_navsat_driver

regul

sbg_ros_driver

velodyne_pointcloud (optional)

velodyne_reader (optional)

rosbridge_server (optional)

ROS packages are on the robot (e.g. installed via apt when available, or as source in the workspace, see Developer information section below). Then:

source devel/setup.bash

export ROS_MASTER_URI=http://192.168.1.200:11311

export ROS_IP=192.168.1.200

roslaunch regul launcher.launch

On the laptop, optionally (to check):

cd Robot_4_roues/

catkin_make

source devel/setup.bash

export ROS_MASTER_URI=http://192.168.1.200:11311

export ROS_IP=192.168.1.185

roslaunch ihm ihm.launch

The Velodyne configuration page can be accessed on http://192.168.1.201. It is physically connected to an Ethernet interface of the embedded computer (via an Ethernet switch) and should be configured to send data only to the embedded computer since the bandwidth would be too limited through Wi-Fi and a broadcast would also slow down the whole network due to its large data.

To test the obstacle detection, try roslaunch velodyne_reader launcher.launch on the embedded computer (the robot should stop and lights should turn on when an obstacle is too close).

To give access to raw sensors and actuators data to another computer without ROS, on the embedded computer (to check):

cd ~/Robot_4_roues

source devel/setup.bash

export ROS_MASTER_URI=http://192.168.1.200:11311

export ROS_IP=192.168.1.200

roslaunch regul raw.launch

Then it should be possible to use the Python package roslibpy on the other computer to e.g. control the motors through /cmd_vel ROS topic, and get sensors data though /imu/data, /fix, /velodyne_points, etc. (see other/roslibpy_sample.py and Developer information section below, especially the Low-level nodes subsection). Note however that the Velodyne data might be too large to be processed remotely (please comment all the lines mentioning velodyne in raw.launch if it is causing issues).

The camera is on http://192.168.1.209 (user: root, password: root), the Wi-Fi AP on http://192.168.1.202 (user: ubnt, password: Robot4roues).

To cleanly shutdown the computer:

sudo shutdown now

MagMap

Warning: always ensure the magnetometer is kept as far away as possible from any potentially magnetic hardware.

The embedded MagMap computer and the magnetometer interface box should be powered through 12 V voltage converters using any Li-Po 3S, 4S or 6S battery of sufficient capacity. They should be installed on the rear of the robot, while the magnetometer and the GPS should be installed on the towed sled using a cable length of 8 m (this is the length configured in the waypoints controller so that the sled follows the correct path).

The embedded MagMap computer (hostname: NUC-12) can be accessed using RDP (mstsc /admin command on Windows, on non-default port 443) or NoMachine (on default port 4000) on the Robot_4_roues Wi-Fi network (see previous section), with 192.168.1.212 IP address (user: Administrator, password: Workstation).

Ensure the GPS is connected in USB, as well as the magnetometer interface box.

Start config_map.vspe (creates 2 bridged virtual serial ports COM15 and COM16 running at a baudrate of 921600).

Launch FGM3D app, ensure it is configured to output data on COM15 with a baudrate of 921600, click Connect (this might need to be retried several times until it succeeds) and then Start and Live streaming\Start.

Start Ubuntu 20.04 virtual machine (user: robin, pass: Robot4roues). On the top-right of VMware Player, ensure there is an indication about COM16 and ublox GNSS receiver connected to the VM (the GPS should appear as /dev/ttyACM0 and will be opened by simple_gnss_node (or nmea_navsat_driver), which assumes that the GPS has been configured to send $GPGGA NMEA sentences).

Then:

sudo chmod a+rw /dev/ttyS1

sudo chmod a+rw /dev/ttyACM0

cd ~/workspace_GNSS

source devel/setup.bash

roslaunch sensys launcher-sensys.launch

A CSV file should appear in ~/.ros. To check data, run in another tab:

rostopic echo /data

Optionally, in another tab:

rosbag record /data

Then the robot can be manually controlled to map the area, or follow predefined waypoints as in the previous section.

Finally, the magnetic map can be displayed by running analyze_map.py on the CSV file.

Developer information

See https://github.com/ENSTABretagneRobotics/SATURNE_simulation, especially saturne branch.

High-level nodes

ihm ROS package:

GUI displaying information from regul package.

regul ROS package:

mission node: specific mission of lines, waypoints, circles.

obj node: line, waypoint, circle following, communication with ihm node using custom messages.

ctrl node: yaw and translation speed control.

estim node: check GPS quality, fuse GPS data with AHRS-based model (without using motor inputs).

velodyne_reader ROS package:

Obstacle detection in Velodyne data.

Low-level nodes

roboteq (needs serial, available on https://github.com/wjwwood/serial ) ROS package:

Low-level control of the motors. 2 instances of the node need to be launched, one for the front Roboteq board controlling the 2 front motors and the other for the rear Roboteq board controlling the 2 rear motors.

Important ROS topics: subscribes to geometry_msgs::Twist /cmd_vel, roboteq::msg_cmd_esc cmd_esc; publishes to std_msgs::Float64 batt_volts, internal_volts, left_amps, right_amps.

light (needs serial, available on https://github.com/wjwwood/serial ) ROS package:

Control of the 2 front white lights and the orange beacon through a Pololu Maestro board. Important ROS topics: subscribes to std_msgs::Bool /pololu/lights_on, /pololu/blinking_on, /pololu/warning_on, /emergency; publishes to: std_msgs::Bool /pololu/button_0, /pololu/button_1, /pololu/button_2.

sbg_driver (could be installed in the system using sudo apt install ros-melodic-sbg-driver) ROS package:

AHRS data.

Important ROS topics: publishes to sensor_msgs::Imu /imu/data.

nmea_navsat_driver (could be installed in the system using sudo apt install ros-melodic-nmea-navsat-driver) ROS package:

GPS data.

Important ROS topics: publishes to sensor_msgs::NavSatFix /fix.

velodyne_pointcloud (could be installed in the system using sudo apt install ros-melodic-velodyne) ROS package:

Velodyne data.

Important ROS topics: publishes to sensor_msgs::PointCloud2 /velodyne_points.

rosbridge_server (could be installed in the system using sudo apt install ros-melodic-rosbridge-suite) ROS package:

Access to ROS topics from other computers without ROS.

Maintenance

Check that the tires are inflated.

Regularly check that all screws are properly tightened.